Grading Scheme

| No. |  |  |  | Scores |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Question A(1) |  | Measure, tabulate, and plot the J vs. I curve. |  | 1.5 pt. |  |  |
| a. |  | Proper data table marked with variables and units. |  | 0.3 |  |  |
| b. |  | Proper sizes of scales, and units for abscissa and ordinate that bear relation to the accuracy and range of the experiment. |  | 0.3 |  |  |
| c. |  | Proper data and adequate curve plotting (Fig. A-1) |  | 0.9 |  |  |
| Question A(2) |  | Estimate the maximum current Im with uncertainty in the linear region of the J vs. I curve. Mark the linear region on the J - I curve figure by using arrows () and determine the threshold current Ith with uncertainty. |  | 3.5 pts. |  |  |
| a. |  | Mark the linear region. |  | 0.5 |  |  |
| b. |  | Least-square fit or eye-balling with ruler and error analysis |  | 1.5 |  |  |
| c. |  | Obtain Im  Im properly |  | 0.5 |  |  |
| d. |  | Adequate value of Ith   Ith |  | 1.0 |  |  |
| No. |  |  | Sub scores |  | Total scores |  |
| B-(1) | Measure, tabulate, and plot the electro-optical switching curve (J vs. Vrms curve) of the NB 90o TN LC, and find its switching slope γ, where γ is defined as (V90–V10)/V10. |  |  |  | 5.0 pts. |  |
|  | - Proper data table marked with variables and units. |  | 0.3 |  |  |  |
|  | - Properly choose the size of scales and units for abscissa and ordinate that bears the relation to the accuracy and range of the experiment. |  | 0.3 |  |  |  |
|  | - Correct measurement of the light intensity (J) as a function of the applied voltage (Vrms) and adequate J - Vrms curve plot. |  |  |  |  |  |
|  | - The intensity of the transmission light reaches zero value in the normally black mode. |  | 0.4 |  |  |  |
|  | - There is a small optical bounce before the external applied voltage reaches the critical voltage. |  | 0.8 |  |  |  |
|  | - The intensity of the transmission light increases rapidly and abruptly when the external applied voltage exceeds the critical voltage. |  | 0.4 |  |  |  |
|  | - The intensity of the transmission light displays the plateau behavior as the external applied voltage exceeds 3.0 Volts. |  | 0.4 |  |  |  |
|  | - Adequate value of  with error,   . |  |  |  |  |  |
|  | - Correctly analyzing the maximum light intensity. |  | 0.6 |  |  |  |
|  | - Correctly analyzing the value of V90. |  | 0.6 |  |  |  |
|  | - Correctly analyzing the value of V10. |  | 0.6 |  |  |  |
|  | - Correct   value, (0.42 ~ 0.44)  0.02. |  | 0.6 |  |  |  |
| B-(2) | Determine the critical voltage Vc of this NB 90o TN LC cell. Show explicitly with graph how you determine the value Vc. |  |  |  | 2.5 pts. |  |
|  | Adequate value of VC with error, VC   VC. |  |  |  |  |  |
|  | - Make the expanded scale plot and take more data points in the region of VC. |  | 0.8 |  |  |  |
|  | - Correctly analyzing the value of VC. |  | 0.7 |  |  |  |
|  | - Correct VC   VC value, (1.2 ~ 1.5)  0.01 Volts. |  | 1.0 |  |  |  |
| C-(1) | Assume that the wavelength of laser light 650 nm, LC layer thickness 7.7 μm, and approximate value of Δn ≈ 0.25 are known. From the experimental data T⊥ and T‖ obtained above, calculate the accurate value of the phase retardation δ and accurate value of birefringence Δn of this LC cell at V=0. |  |  |  | 2.5 pts. |  |
|  | Adequate value of  and n with error. |  |  |  |  |  |
|  | - Correctly analyzing the values of T‖. |  | 0.3 |  |  |  |
|  | - Correctly analyzing the values of T⊥. |  | 0.3 |  |  |  |
|  | - Correctly determining the value of order m. |  | 0.9 |  |  |  |
|  | - Correct value, 17.7 ~ 18.2. |  | 0.5 |  |  |  |
|  | - Correct n value, 0.23 ~ 0.25. |  | 0.5 |  |  |  |
| C-(2) | Measure, tabulate, and plot the electro-optical switching curve for T‖ of this parallel aligned LC cell in the θ = 45o configuration. |  |  |  | 3.0 pts. |  |
|  | - Proper data table marked with variables and units. |  | 0.3 |  |  |  |
|  | - Properly choose the size of scales and units for abscissa and ordinate that bears the relation to the accuracy and range of the experiment. |  | 0.3 |  |  |  |
|  | - Correct measurement of the T‖ as a function of the applied voltage (Vrms) and adequate T‖-Vrms curve plot. |  |  |  |  |  |
|  | - Three minima and two sharp maxima. |  | 1.5 |  |  |  |
|  | - Maxima values within 15 % from each other. |  | 0.5 |  |  |  |
|  | - Minima are less than the values of 0.1 Volts. |  | 0.4 |  |  |  |
| C-(3) | From the electro-optical switching data, find the value of the external applied voltage Vπ |  |  |  | 2.0 pts. |  |
|  | Adequate value of Vπ with error. |  |  |  |  |  |
|  | - Make the expanded scale plot and take more data points in the region of Vπ. |  | 0.3 |  |  |  |
|  | - Indicate the correct minimum of Vπ. |  | 0.8 |  |  |  |
|  | - Correctly analyzing the value of Vπ. |  | 0.5 |  |  |  |
|  | - Correct Vπ  Vπvalue, (3.2 ~ 3.5 )  0.1 Volts. |  | 0.4 |  |  |  |

Solutions

(Part A) Laser diode and Photodetector

Question A-(1) (Total 1.5 point)

Measure, tabulate, and plot the J vs. I curve.

a. Data (0.3 pts.); Proper data table marked with variables and units.

Table A-(1): Data for J vs. I.

| I (mA) | 9.2 | 15.2 | 19.5 | 21.6 | 22.2 | 22.7 | 23.0 | 23.4 | 23.8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J (V) | 0.00 | 0.01 | 0.02 | 0.03 | 0.05 | 0.06 | 0.09 | 0.12 | 0.30 |
| I (mA) | 24.2 | 24.6 | 25.0 | 25.4 | 25.8 | 26.2 | 26.6 | 27.0 | 27.4 |
| J (V) | 0.66 | 1.02 | 1.41 | 1.88 | 2.23 | 2.64 | 3.04 | 3.36 | 3.78 |
| I (mA) | 27.8 | 28.2 | 28.6 | 29.0 | 29.4 | 29.8 | 30.2 | 30.5 | 31.0 |
| J (V) | 4.12 | 4.48 | 4.79 | 5.13 | 5.44 | 5.72 | 6.05 | 6.25 | 6.55 |
| I (mA) | 31.4 | 31.8 | 32.2 | 32.6 | 33.0 | 33.4 | 33.8 | 34.2 | 34.6 |
| J (V) | 6.75 | 6.99 | 7.22 | 7.40 | 7.60 | 7.78 | 7.93 | 8.07 | 8.14 |
| I (mA) | 35.0 | 35.5 | 36.0 | 36.5 | 37.0 | 37.6 | 38.0 | 38.6 |  |
| J (V) | 8.18 | 8.20 | 8.22 | 8.24 | 8.24 | 8.25 | 8.26 | 8.27 |  |

Current error: 0.1 mA, Voltage: 0.01 V

b. Plotting (0.3 pts.): Proper sizes of scales, and units for abscissa and ordinate that bear relation to the accuracy and range of the experiment.

c. Curve (0.9 pts.): Proper data and adequate line shape

‧ As shown in Fig. A-1. Start ~0 → Threshold → Linear → Saturate

Im   Im

Fig. A-1    J vs. I graph

Question A-(2) ( Total 3.5 points)

Estimate the maximum current Im with uncertainty in the linear region of the J - I. Mark the linear region on the J - I curve figure by using arrows () and determine the threshold current Ith with detailed error analysis.

a. Linear region marking (0.5 pts.) in Fig. A-1

b. Least-square method or eye-balling with ruler and error analysis (1.5 pt.)

| Least-square Fitting | eye-balling with ruler |
| --- | --- |
| Error bar in graph 0.0x mA (0.5 pts) | Error bar in graph 0.x mA (0.5 pts) |
| Least-square method (0.5 pts) | Expanded scale graph (0.5 pts) |
| Error analysis (0.5 pts) | draw three lines for error analysis(0.5 pts) |

c. Im   Im (0.5 pts.): Adequate value of Im (0.3 pts.) and error(  Im ) (0.2 pts.) from the linear region of J-I curve.

- Adequate value of Ith with error (1.0 pts.)

Ith = (21~26)  (0.01 or 0.2 for single value) mA

Adequate value of Ith (0.5 pts.) and error (  Ith ) (0.5 pts.)

Fig. A-2 Straight lines and extrapolations.

Appendix

◎A1-1

‧Least-square：

I=m J +b   →   b=Ith

For y=mx+b

|  | Y:I(mA) | X: J | XY | X2 | Y(X)=mX+b | (Y-Y(X))2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 23.8 | 0.3 | 7.14 | 0.09 | 23.7937 | 3.969E-05 |
| 2 | 24.2 | 0.66 | 15.972 | 0.4356 | 24.17134 | 0.000821 |
| 3 | 24.6 | 1.02 | 25.092 | 1.0404 | 24.54898 | 0.00260 |
| 4 | 25 | 1.41 | 35.25 | 1.9881 | 24.95809 | 0.00176 |
| 5 | 25.4 | 1.88 | 47.752 | 3.5344 | 25.45112 | 0.00261 |
| 6 | 25.8 | 2.23 | 57.534 | 4.9729 | 25.81827 | 0.000334 |
| 7 | 26.2 | 2.64 | 69.168 | 6.9696 | 26.24836 | 0.00234 |
| 8 | 26.6 | 3.04 | 80.864 | 9.2416 | 26.66796 | 0.00462 |
| 9 | 27 | 3.36 | 90.72 | 11.2896 | 27.00364 | 1.325E-05 |
| 10 | 27.4 | 3.78 | 103.572 | 14.2884 | 27.44422 | 0.00196 |
| 11 | 27.8 | 4.12 | 114.536 | 16.9744 | 27.80088 | 7.744E-07 |
| 12 | 28.2 | 4.48 | 126.336 | 20.0704 | 28.17852 | 0.000461 |
| 13 | 28.6 | 4.79 | 136.994 | 22.9441 | 28.50371 | 0.00927 |
|  | Y = 340.6 | X= 33.71 | XY= 910.93 |  Y2 = 113.840 |  |  (Y-Y(x))2 = 0.0268 |

Δ=NΣx2−Σx2=13(113.840)−(33.71)2=343.556m=1ΔNΣxy−ΣxΣy=13(910.93)−(33.71)(340.6)343.556=1.049b=1ΔΣx2Σy−ΣxΣxy=(113.840)(340.6)−(33.71)(910.93)343.556=23.479

σy=1N−2Σy−y(x)2=113−20.0268=0.015σ=(σy)2+dydxσx2=(0.015)2+1.049×0.0052=0.016σm=Nσ2Δ=13×0.0162343.556=0.0031 σb=σ2ΔΣx2=0.016×113.840343.556=0.0092

Ith=23.48±0.01 mA

◎A1-2

‧Eye-balling：

I=m J +b   →   b=Ith

For y=mx+b

Line 1: Y = 1.00X + 23.66

Line 2: Y = 1.05X + 23.48

Line3: Y = 1.13X + 23.31

Ith(av.) = 23.48

Ith(std.) = 0.18

Ith=23.5±0.2 mA

Solutions

Question B-(1) (Total 5.0 points)

Measure, tabulate, and plot the electro-optical switching curve (J vs. Vrms curve) of the NB 90o TN LC, and find its switching slope γ, where γ is defined as (V90–V10)/V10.

- Proper data table marked with variables and units. (0.3 pts)

| Applied voltage (Volts) | Light intensity (Volts) | Applied voltage (Volts) | Light intensity (Volts) |
| --- | --- | --- | --- |
| 0.00 | 0.00 | 2.44 | 1.22 |
| 0.10 | 0.00 | 2.50 | 1.26 |
| 0.20 | 0.00 | 2.55 | 1.27 |
| 0.30 | 0.00 | 2.60 | 1.29 |
| 0.40 | 0.00 | 2.67 | 1.32 |
| 0.50 | 0.00 | 2.72 | 1.33 |
| 0.60 | 0.00 | 2.85 | 1.36 |
| 0.70 | 0.00 | 2.97 | 1.37 |
| 0.80 | 0.00 | 3.11 | 1.38 |
| 0.90 | 0.00 | 3.20 | 1.39 |
| 1.00 | 0.00 | 3.32 | 1.39 |
| 1.10 | 0.02 | 3.41 | 1.39 |
| 1.20 | 0.04 | 3.50 | 1.40 |
| 1.24 | 0.04 | 3.60 | 1.39 |
| 1.30 | 0.04 | 3.70 | 1.40 |
| 1.34 | 0.03 | 3.80 | 1.40 |
| 1.38 | 0.02 | 4.03 | 1.40 |
| 1.45 | 0.01 | 4.22 | 1.40 |
| 1.48 | 0.01 | 4.40 | 1.39 |
| 1.55 | 0.02 | 4.61 | 1.39 |
| 1.59 | 0.03 | 4.78 | 1.40 |
| 1.64 | 0.05 | 5.03 | 1.39 |
| 1.71 | 0.11 | 5.20 | 1.39 |
| 1.78 | 0.21 | 5.39 | 1.38 |
| 1.81 | 0.26 | 5.61 | 1.39 |
| 1.85 | 0.33 | 5.81 | 1.38 |
| 1.90 | 0.44 | 6.02 | 1.38 |
| 1.96 | 0.57 | 6.21 | 1.38 |
| 2.03 | 0.70 | 6.40 | 1.38 |
| 2.08 | 0.80 | 6.63 | 1.38 |
| 2.15 | 0.92 | 6.80 | 1.38 |
| 2.21 | 1.02 | 7.02 | 1.38 |
| 2.28 | 1.10 | 7.20 | 1.38 |
| 2.33 | 1.14 |  |  |
| 2.39 | 1.19 |  |  |

- Properly choose the size of scales and units for abscissa and ordinate that bears the relation to the accuracy and range of the experiment. (0.3 pts)

- Correct measurement of the light intensity (J) as a function of the applied voltage (Vrms) and adequate J - Vrms curve plot.

  - The intensity of the transmission light is smaller than 0.05 Volts in the normally black mode. (0.4 pts)

  - There is a small optical bounce before the external applied voltage reaches the critical voltage. (0.8 pts)

  - The intensity of the transmission light increases rapidly and abruptly when the external applied voltage exceeds the critical voltage. (0.4 pts)

  - The intensity of the transmission light displays the plateau behavior as the external applied voltage exceeds 3.0 Volts. (0.4 pts)

- Adequate value of  with error.

  - Find the maximum value of the light intensity in the region of the applied voltage between 3.0 and 7.2 Volts (0.6 pts)

  - Determine the value of 90 % of the maximum light intensity. Obtain the value of the applied voltage V90 by interpolation. (0.6 pts)

  - Determine the value of 10 % of the maximum light intensity. Obtain the value of the applied voltageV10 by interpolation. (0.6 pts)

  - Correct   value, (0.42 ~ 0.44)  0.02. (0.4+0.2 pts)

Question B-(2) (Total 2.5 points)

Determine the critical voltage Vc of this NB 90o TN LC cell.  Show explicitly with graph how you determine the value Vc.

  - Adequate value of VC with error, VC   VC.

    - Make the expanded scale plot and take more data points in the region of VC. (0.8 pts)

    - Determine the value of VC when the intensity of the transmission light increases rapidly and abruptly. (0.7 pts)

    - Correct VC   VC value, (1.20 ~ 1.50)  0.01 Volts. (0.8+0.2 pts)

( The data shown in this graph do not correspond to the data shown on page 9.  This graph only shows how to obtain Vc. )

Question C-(1) (2.5 points)

Assume that the wavelength of laser light 650 nm, LC layer thickness 7.7 μm, and approximate value of Δn ≈ 0.25 are known. From the experimental data T⊥ and T‖obtained above, calculate the accurate value of the phase retardation δ and accurate value of birefringence Δn of this LC cell at V=0.

a. Adequate value of  and n with error.

  - Take and average the values of T‖. (0.3 pts)

  - Take and average the values of T⊥. (0.3 pts)

  - Determine the value of order m.  (0.9 pts)

  - Correct value, 15.7 ~ 18.2.  (0.5 pts)

  - Correct n value, 0.20 ~ 0.24  (0.5 pts)

T//=0.31+0.31+0.313=0.31±0.01 Volts

T=1.04+1.03+1.043=1.04±0.01 Volts

tanδ2=±TT//=−1.83  ∴δ=4.14+2mπ(or−2.14+2mπ)

δ=2πdΔnλ=2π×7.7×0.250.65=18.61

Take m=2(or3)	∴δ=16.70(5.32π)

From δ=2πdΔnλ ∴Δn=δλ2πd=0.22

Accepted value for ∴Δn=(0.20−0.24)

*if tanδ2=1.83, the value for δ will be either 4.68π or 6.68π,

which is not consistent with data figure of problem C2.

Question C-(2) (Total 3.0 points)

Measure, tabulate, and plot the electro-optical switching curve for T‖ of this parallel aligned LC cell in the θ = 45o configuration.

a. Proper data table marked with variables and units. (0.3 pts)

| Applied voltage (Volts) | Light intensity (Volts) | Applied voltage (Volts) | Light intensity (Volts) | Applied voltage (Volts) | Light intensity (Volts) |
| --- | --- | --- | --- | --- | --- |
| 0.00 | 0.30 | 1.80 | 0.78 | 2.85 | 0.21 |
| 0.10 | 0.30 | 1.83 | 0.92 | 2.88 | 0.18 |
| 0.20 | 0.29 | 1.86 | 1.05 | 2.91 | 0.16 |
| 0.30 | 0.29 | 1.89 | 1.19 | 2.94 | 0.14 |
| 0.40 | 0.29 | 1.92 | 1.27 | 2.97 | 0.12 |
| 0.50 | 0.28 | 1.95 | 1.34 | 3.00 | 0.09 |
| 0.60 | 0.26 | 1.98 | 1.40 | 3.06 | 0.08 |
| 0.70 | 0.23 | 2.01 | 1.47 | 3.09 | 0.06 |
| 0.80 | 0.19 | 2.04 | 1.48 | 3.12 | 0.05 |
| 0.90 | 0.09 | 2.07 | 1.48 | 3.18 | 0.04 |
| 0.99 | 0.00 | 2.10 | 1.48 | 3.21 | 0.03 |
| 1.02 | 0.06 | 2.13 | 1.45 | 3.24 | 0.02 |
| 1.05 | 0.16 | 2.16 | 1.42 | 3.27 | 0.02 |
| 1.08 | 0.25 | 2.19 | 1.38 | 3.30 | 0.01 |
| 1.11 | 0.40 | 2.22 | 1.33 | 3.33 | 0.00 |
| 1.14 | 0.67 | 2.25 | 1.27 | 3.36 | 0.00 |
| 1.17 | 0.93 | 2.28 | 1.20 | 3.39 | 0.00 |
| 1.20 | 1.25 | 2.31 | 1.14 | 3.42 | 0.00 |
| 1.26 | 1.31 | 2.34 | 1.07 | 3.45 | 0.00 |
| 1.29 | 1.36 | 2.37 | 1.00 | 3.48 | 0.00 |
| 1.32 | 1.32 | 2.40 | 0.94 | 3.51 | 0.00 |
| 1.35 | 1.09 | 2.43 | 0.87 | 3.60 | 0.01 |
| 1.38 | 0.85 | 2.46 | 0.79 | 3.70 | 0.02 |
| 1.41 | 0.62 | 2.49 | 0.72 | 3.80 | 0.03 |
| 1.44 | 0.46 | 2.52 | 0.66 | 3.90 | 0.04 |
| 1.47 | 0.29 | 2.55 | 0.61 | 4.00 | 0.07 |
| 1.50 | 0.13 | 2.58 | 0.56 | 4.10 | 0.09 |
| 1.53 | 0.06 | 2.61 | 0.51 | 4.20 | 0.11 |
| 1.59 | 0.03 | 2.64 | 0.46 | 4.30 | 0.14 |
| 1.62 | 0.05 | 2.67 | 0.42 | 4.40 | 0.16 |
| 1.65 | 0.15 | 2.70 | 0.37 | 4.50 | 0.19 |
| 1.68 | 0.24 | 2.73 | 0.33 | 4.60 | 0.22 |
| 1.71 | 0.34 | 2.76 | 0.30 | 4.70 | 0.25 |
| 1.74 | 0.49 | 2.79 | 0.26 | 4.80 | 0.28 |
| 1.77 | 0.63 | 2.82 | 0.23 | 4.90 | 0.31 |

| Applied voltage (Volts) | Light intensity (Volts) |
| --- | --- |
| 5.01 | 0.34 |
| 5.11 | 0.37 |
| 5.21 | 0.39 |
| 5.29 | 0.42 |
| 5.39 | 0.44 |
| 5.51 | 0.48 |
| 5.57 | 0.49 |
| 5.70 | 0.52 |
| 5.80 | 0.55 |
| 5.90 | 0.57 |
| 6.01 | 0.60 |
| 6.10 | 0.62 |
| 6.19 | 0.64 |
| 6.30 | 0.66 |
| 6.40 | 0.69 |
| 6.60 | 0.73 |
| 6.70 | 0.74 |
| 6.80 | 0.76 |
| 7.00 | 0.80 |
| 7.20 | 0.83 |

- Properly choose the size of scales and units for abscissa and ordinate that bears the relation to the accuracy and range of the experiment. (0.3 pts)

- Correct measurement of the T‖ as a function of the applied voltage (Vrms) and adequate T‖-Vrms curve plot.

  - Three minima and two sharp maxima. (1.5 pts)

  - Maxima values within 15% from each other. (0.5 pts)

  - Minima are less than the values of 0.1 Volts. (0.4 pts)

Question C-(3) (Total 2.0 points)

From the electro-optical switching data, find the value of the external applied voltage Vπ

a. Adequate value of Vπ with error.

- Make the expanded scale plot and take more data points in the region of Vπ. (0.3 pts)

- Indicate the correct minimum of Vπ. (0.8 pts)

- Obtain the value of Vπ by interpolation or rounding. (0.5 pts)

- Correct Vπ value : (3.2 ~ 3.5)±0.01Volts. (0.2+0.2 pts)