Question “Orange”

1.1)

First of all, we use the Gauss’s law for a single plate to obtain the electric field,

E=σε0.

The density of surface charge for a plate with charge,Q and area,A is

σ=QA.

Note that the electric field is generated by two equivalent charged parallel plates. The contribution of each plate to the electric field is 12E. Force is defined by the electric filed times the charge, then we have

Force = 12 E Q = Q22ε0A

1.2)

The Hook’s law for a spring is

Fm=−k x.

In 1.2 we derived the electric force between two plates is

Fe=Q22ε0A.

The system is stable. The equilibrium condition yields

Fm=Fe,

⇒ x=Q22ε0A k

1.3)

The electric field is constant thus the potential difference, V is given by

V=E(d−x)

(Other reasonable approaches are acceptable. For example one may use the definition of capacity to obtainV.)

By substituting the electric field obtained from previous section to the above equation, we get,  V=Q dε0A1−Q22ε0A k d

1.4)

C is defined by the ratio of charge to potential difference, then

C= QV .

Using the answer to 1.3, we get CC0= 1−Q22ε0A k d−1

1.5)

Note that we have both the mechanical energy due to the spring

Um=12kx2,

and the electrical energy stored in the capacitor.

UE=Q22C .

Therefore the total energy stored in the system is

U=Q2d2ε0A1−Q24ε0A k d

2.1)

For the given value of x, the amount of charge on each capacitor is

Q1=V C1=ε0A Vd−x ,

Q2=V C2=ε0A Vd+x .

2.2)

Note that we have two capacitors. By using the answer to 1.1 for each capacitor, we get

F1=Q122ε0A,

F2=Q222ε0A.

As these two forces are in the opposite directions, the net electric force is

FE=F1−F2,   ⇒ FE=ε0A V221(d−x)2−1(d+x)2

2.3)

Ignoring terms of order x2 in the answer to 2.2., we get

FE=2ε0A V2d3x

2.4)

There are two springs placed in series with the same spring constant,k, then the mechanical force is

Fm=−2 k x.

Combining this result with the answer to 2.4 and noticing that these two forces are in the opposite directions, we get

F=Fm+FE,                  ⇒ F= −2k−ε0A V2d3 x,

⇒ keff=2k−ε0A V2d3

2.5)

By using the Newtown’s second law,

F=ma

and the answer to 2.4, we get

a=−2mk−ε0A V2d3 x

3.1)

Starting with Kirchhoff’s laws, for two electrical circuits, we have

QSCS+V−Q2C2=0−QSCS+V−Q1C1=0Q2−Q1+QS=0

Noting that Vs= QSCS  one obtains

⇒ VS=V2ε0A xd2−x2CS+2ε0A dd2−x2 .

Note: Students may simplify the above relation using the approximationd2>>x2. It does not matter in this section.

3.2)

Ignoring terms of order x2 in the answer to 3.1., we get

VS=V2ε0A xd2CS+2ε0A d .

4.1)

The ratio of the electrical force to the mechanical (spring) force is

FEFm=ε0A V2keff d3 ,

Putting the numerical values:

FEFm=7.6×10−9 .

As it is clear from this result, we can ignore the electrical forces compared to the electric force.

4.2)

As seen in the previous section, one may assume that the only force acting on the moving plate is due to springs:

F=2k x .

Hence in mechanical equilibrium, the displacement of the moving plate is

x=ma2 k .

The maximum displacement is twice this amount, like the mass spring system in a gravitational force field, when the mass is let to fall.

xmax=2 x

xmax= m ak

4.3)

At the acceleration

a=g,

The maximum displacement is

xmax= m gk.

Moreover, from the result obtained in 3.2, we have

VS=V2ε0A xmaxd2CS+2ε0A d

This should be the same value given in the problem,0.15 V.

⇒ CS=2ε0AdV xmaxVS d−1

⇒ CS=8.0×10−11 F

4.4)

Let ℓ be the distance between the driver’s head and the steering wheel. It can be estimated to be about

ℓ=0.4 m− 1 m.

Just at the time the acceleration begins, the relative velocity of the driver’s head with respect to the automobile is zero.

Δv(t=0)=0,

then

ℓ=12g t12     ⇒ t1=2 ℓg

t1=0.3−0.5 s

4.5)

The time t2is half of period of the harmonic oscillator, hence

t2= T2,

The period of harmonic oscillator is simply given by

T=2πmk ,

therefore,

t2=0.019 s.

As t1> t2, the airbag activates in time.