NAME:									CODE

Physics Laboratory practical

Total marks [12.6]

## Title: Determination of coefficient of viscosity of oil

## Determination of coefficient of viscosity of fluids

## Ball Drop Experiment

The measurement involves determining the velocity of a falling sphere through a column of fluid of unknown viscosity. This is accomplished by dropping a sphere through a measured distance of fluid and measuring how long it takes to traverse the distance.

## Materials

- Thermometer

- Ball bearings of different diameters

- Stopwatch

- Meter ruler

- Paper towel

- Magnet

- Oil

## Theoretical aspects

Consider a spherical ball bearing of radius r and density ρs falling through a column of viscous fluid of coefficient of viscosity  and density  ρf as illustrated in the figure below. The coefficient of viscosity is a measure of the degree of internal resistance to flow and shear.

Figure 1: showing a sphere of radius r falling through a column of fluid of density ρf. A and B marks the distance travelled by the sphere at terminal velocity vt.

According to Newton’s second law:

Net Force=ma

ma=W-Fu+Fv                         (1)

Where m is the mass of the sphere,

W=mg, is the weight of the sphere (ball bearing)

Fu= 43πr3ρfg is the upthrust = weight of the fluid displaced

Fv=6 π r η v is the viscous force (of a sphere of radius r) proportional to the velocity v of the ball (Stoke’s Law).

Initially the ball has some downward acceleration until the sphere acquires terminal velocity vt (vt=st where s is the distance travelled in time t), when there is no more acceleration and hence the net force is zero. Equation (1) becomes

mg= Fu+Fv

43π r3ρs g= 43π r3ρf g+ 6 π r η vt               (2)

Or vt= 29r2η g ρs- ρf                            (3)

Note that s is the distance between A and B and t is the time the ball takes to fall between A and B.

Equation  (3) can be modified to:

vt= 118d2η g ρs- ρf                             (4)

Where

d = diameter of sphere (=2r)

ρs = density of sphere = m/V = (mass of sphere/volume of sphere)

ρf = density of fluid

g = acceleration of gravity = 9.81 m/s2

vt = Terminal Velocity = s/t = (distance sphere falls)/(time of it takes to fall)

## Procedure

Proceed as follows.

- Measure the vertical distance s between points A and B marked on the cylindrical tubes.

- Drop one of the ball bearings into the fluid (ensuring that the ball bearing does not touch the wall of the cylinder during its motion between A and B)

- Measure the time t taken by the sphere to travel the distance s between A and B and record it in the provided table.

- Without removing the ball bearing, repeat steps 2 and 3 above using other bearings of the same diameter to have three values of time.

- Repeat steps 2 to 4 for the other 4 sizes of ball bearings.

Results and analysis

Note the following:

ρf= 871.4 kg/m3

ρs= 7717 kg/m3

s =   0.4 m     (distance between A and B)

Calculate the average time, d2 and vt for each set of ball bearings, complete Table 1.

Table 1:  experimental results.                                    [2.4]

| Ball diameter |  |  | Diameter squared | Time taken to fall distance l |  |  |  | Terminal velocity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | d (mm) | d (m) | d2 (m²) | t1 (s) | t2 (s) | t3 (s) | Average time (s) | vt (m/s) |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |

- Plot a graph of vt vs d2,                                        [5.2]

- Use the graph to determine the viscosity of the oil with the appropriate units.                                                  [5]

MARKING SCHEME Solution

Completed table

Note the following:

ρf= 871.4 kg/m3

ρs= 7717 kg/m3

Temperature before Tb:

T=										[0.15]

Distance l

l≈0.500 m     								[0.25]

Drawing a sketch	of measurement					[0.75]

Temperature before Ta:

Ta=										[0.15]

Points for the measurements and calculations

Table III-1

| # | (mm). | (m) | (m²) | (s) | (s) | (s). | Average time (s). | (m/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0,25 | 0,05 | 0,05 | 0,25 | 0,25 | 0,25 | 0,05 | 0,1 |
| 2 | 0,25 | 0,05 | 0,05 | 0,25 | 0,25 | 0,25 | 0,05 | 0,1 |
| 3 | 0,25 | 0,05 | 0,05 | 0,25 | 0,25 | 0,25 | 0,05 | 0,1 |
| 4 | 0,25 | 0,05 | 0,05 | 0,25 | 0,25 | 0,25 | 0,05 | 0,1 |

Subtraction (per column) 0.2 if out of range

| d (mm) | Range d (mm) | dmin (mm) | dmax (mm) |
| --- | --- | --- | --- |
| 4,50 | 0,4 | 4,30 | 4,70 |
| 5,00 | 0,4 | 4,80 | 5,20 |
| 6,00 | 0,4 | 5,80 | 6,20 |
| 7,00 | 0,4 | 6,80 | 7,20 |

III-2 Plot of vt vs. d2                                                      [3.0]

Marks allocation:

- x and y axis labelling 				[1.0]

(for each axis Quantity (0.25) & Unit (0.25))

- Scale of the graph				[1.0]

0.5 for each axis (uniform & size)

- Plotting of points (0.1 for each) 		[0.4]

- Drawing straight line best of fit 		[0.6]

III-3 Determining the slope of the line				[1.5]

- Mark the points on the line that are used		[0.5]

- Calculation of the slope				[0.5]

- Determining the correct unit				[0.5]

III-4 Derive and show analytical expression for C		[1.0]

III-5 Determination of viscosity of oil				[1.5]

Mark allocation

- Determination of coefficient of viscosity.

From        vt= 118d2η g ρs- ρf

slope=g18ρs- ρfη     or     slope=Cη    [0.5]

Calculation of the viscosity				[0.5]

Determining the correct unit				[0.5]

η=g18ρs- ρfslope→ ms2*kgm3msm2  = Nsm2=Pa.S